|  | Cat.6 U/UTP 4P PVC-Eca |
| --- | --- |
| DESIGN - Conductor: Solid annealed copper - Insulation: HDPE - Pair: two conductors twisted together Color identification of pairs: - pair 1: white-blue / blue - pair 2: white-orange / orange - pair 3: white-green / green - pair 4: white-brown / brown - Cross-web: HDPE, 2,6x2,6 mm - Sheath: PVC, nominal thickness 0,5 mm |  |
| APPLICATION PBX, V.11, X.21, ISDN, Ethernet (10Base-T), ATM-25/52/155 Mbit/s, 100VG-AnyLAN, Token Ring 16/100 Mbit/s, Fast Ethernet (100BASE- TX), 1000BASE-T, 2.5GBASE-T, 5GBASE-T, PoE, PoE+, PoE++ | INSTALLATION AND OPERATION CONDITIONS For stationary installation inside buildings, stations, structures and equipment. Op- erated on frequencies up to 250 MHz |
| STANDARDS Cabling: ISO/IEC 11801-1, EN 50173-1, ANSI/TIA-568.2-D Cable: IEC 61156-5, EN 50288-6-1, ANSI/TIA-568.2-D Fire safety: - Flame propagation: EN 60332-1-2 CPR class: Eca | MECHANICAL CHARACTERISTICS Temperature range: - During installation -10 °C ... +60 °C - After installation -20 °C ... +60 °C Bending radius: - During installation ≥ 8 cable diameters - After installation ≥ 4 cable diameters Tensile stress ≤ 85 N |

# ELECTRICAL CHARACTERISTICS AT 20 °C

| Parameter | Unit | Value |
| --- | --- | --- |
| Capacitance | pF/m | ≤ 56 |
| Resistance for conductor Ø, mm: - 0,54 | Оhm/km | ≤ 82 |
| Insulation resistance | GOhm*km | ≥ 5 |
| Resistance unbalance | % | 1 |
| Velocity of propagation (NVP) | % | 70 |
| Propagation delay | ns/100 m | ≤ 534+36/√f |
| Delay skew | ns/100 m | ≤ 20 |
| Characteristic impedance in the frequency range of: - 1-100 MHz - 101-250 MHz | Ohm | 100±15 100±22 |
| Test voltage between cores, (DC, 2 s) | kV | 2,5 |

| Frequency, MHz | 1 | 4 | 10 | 16 | 20 | 31,25 | 62,5 | 100 | 250 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attenuation, dB, max | 2,1 | 3,8 | 6 | 7,6 | 8,5 | 10,8 | 15,5 | 19,9 | 33 |
| NEXT, dB, min | 75,3 | 66,3 | 60,3 | 57,3 | 55,8 | 52,8 | 48,3 | 45,3 | 39,3 |
| PS NEXT, dB, min | 72,3 | 63,3 | 57,3 | 54,3 | 52,8 | 49,8 | 45,3 | 42,3 | 36,3 |
| ELFEXT, dB, min | 68 | 56 | 48 | 44 | 42 | 38 | 32 | 28 | 20 |
| PS ELFEXT, dB, min | 65 | 53 | 45 | 41 | 39 | 35 | 29 | 25 | 17 |
| RL, dB, min | 20,0 | 23,0 | 25,0 | 25,0 | 25,0 | 23,6 | 21,5 | 20,1 | 17,3 |

IEC 61156-5, values lower than 4 MHz are given for information only

# ORDER INFORMATION

| P/N | Cable name | Nominal conductor Ø, mm | Nominal insulation Ø, mm | Estimated сable Ø, mm | Standard packaging | Weight, kg/km |
| --- | --- | --- | --- | --- | --- | --- |
| 49205 | KPV-VP (250) 4*2*0,54 (U/UTP-cat.6) | 0,54 | 0,98 | 5,745 | Cardboard box 305 m | 35,8 |