|  | F/UTP cat.6 4x2x0,54 LSZH |
| --- | --- |

DESIGN

- Conductor: soft copper wire, nominal Ø 0,54 mm

- Insulation: PE, Ø 1,09 mm

- Pair: two conductors twisted together

- Cross-web: PE, 4x4 mm

- Core wrapping

- Copper tinned contact wire Ø 0,4 mm

- Core shield: aluminum polymer foil (metal outside)

- Sheath: LSZH, nominal cable diameter: 6,8 mm

| APPLICATION PBX, V.11, X.21, ISDN, Ethernet (10Base-T), ATM-25/52/155 Mbit/s, 100VG-AnyLAN, Fast Ethernet (100BASE-TX), Token Ring 16/100 Mbit/s, Gigabit Ethernet (1000BASE-T), Firewire 100 Mbit/s, 10 Gigabit Ethernet (10GBASE-T*), ATM LAN 1,2 Gbit/s | INSTALLATION AND OPERATION CONDITIONS For stationary installation inside buildings, stations, structures and equipment in conditions of heightened electromagnetic impacts. Operated on frequencies up to 250 MHz. |
| --- | --- |
| СOMPLIANCE WITH THE REQUIREMENTS System: ISO/IEC 11801, EN 50173-1, ANSI/TIA 568-С.2 Cable: IEC 61156-5 Fire safety: IEC 60332-1, IEC 61034-1, IEC 60754-2 CPR class: Eca | MECHANICAL CHARACTERISTICS Тemperature range: During installation -10 °C ... +60 °C After installation -20 °C ... +60 °C Bending radius: During installation ≥ 8 cable diameters After installation ≥ 4 cable diameters Tensile stress ≤ 85 N |

# ELECTRICAL CHARACTERISTICS AT 20 °C

| Parameter | Unit | Value |
| --- | --- | --- |
| Capacitance | pF/m | ≤ 56 |
| Direct current resistance | Оhm/km | ≤ 85 |
| Insulation resistance | GOhm/km | ≥ 5 |
| Resistance unbalance | % | 2 |
| Signal propagation velocity | s | 0,68 |
| Propagation delay | ns/100 m | ≤ 534+36/√f |
| Delay shift on the frequency 100 MHz | ns/100 m | ≤ 30 |
| Characteristic impedance in the frequency range of: 1-100 MHz 100-250 MHz | Ohm | 100±15 100±22 |
| Test voltage between cores, (DC, 2 s) | kV | 2,5 |

| Frequency, MHz | 1 | 4 | 10 | 16 | 20 | 31,25 | 62,5 | 100 | 125 | 200 | 250 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attenuation, dB, max | 2,1 | 3,8 | 6 | 7,6 | 8,5 | 10,8 | 15,5 | 19,9 | 22,5 | 29,1 | 33 |
| NEXT, dB, min | 75,3 | 66,3 | 60,3 | 57,3 | 55,8 | 52,8 | 48,3 | 45,3 | 43,8 | 40,8 | 39,3 |
| PS NEXT, dB, min | 72,3 | 63,3 | 57,3 | 54,3 | 52,8 | 49,8 | 45,3 | 42,3 | 40,8 | 37,8 | 36,3 |
| ELFEXT, dB, min | 68 | 56 | 48 | 44 | 42 | 38 | 32 | 28 | 26 | 22 | 20 |
| PS ELFEXT, dB, min | 65 | 53 | 45 | 41 | 39 | 35 | 29 | 25 | 23 | 19 | 17 |
| RL, dB, min | 20,0 | 23,0 | 25,0 | 25,0 | 25,0 | 23,6 | 21,5 | 20,1 | 19,4 | 18,0 | 17,3 |

IEC 61156-5, values lower than 4 MHz are given for information only